FIBER SPLICE RESTORATION — SPRINGFIELD BUSINESS PARK

Status: OPEN  ·  Priority: CRITICAL  ·  Due: 2026-05-20T17:00:00Z

| Assigned Field Technician | On-Site Technician | Location | Created |
| --- | --- | --- | --- |
| John Smith | — | 742 Evergreen Terrace, Springfield, WA 99999 | 2026-05-18 08:15 PDT |

| Dispatch Log | 2026-05-18 08:15 PDT ¦ NOC Ref: WO-DISP-0518 ¦ Dispatcher: R. Singh ¦ Route → J. Martinez ¦ Status: Pending Accept |
| --- | --- |

## Job Description

Restore 24-count feeder splice after cabinet damage and verify light levels for affected tenants. A utility vehicle struck the fiber cabinet at the northwest corner of the building parking structure, shearing the 24-count SM feeder at approximately 3 m from the splice enclosure. Fourteen tenant circuits are currently down. Technician must expose the damaged section, re-splice all affected fibers, reseal the enclosure, and confirm restore of signal for each affected circuit before closure.

## Site Access & Contact

Contact: John Smith — Network Operations Supervisor  |  (425) 555-0183

Access Notes: Escort required. Check in at security desk (lobby entrance). Hard hat + safety vest mandatory in parking structure.

## Safety Protocols

Wear PPE: safety glasses, cut-resistant gloves, and high-vis vest

Lock out / tag out power to cabinet before opening enclosure

Confirm no live laser sources before handling bare fiber

Dispose of fiber cleave scraps in designated sharps container

## Parts & Materials Required

| Part ID | Description | Qty |
| --- | --- | --- |
| FIB-003 | Single-Mode Splice Tray (12-fiber) | 2 |
| FIB-012 | Fiber Splice Enclosure (24-count) | 1 |

## Field Completion Checklist

Review as-built drawings for cable route

Confirm part availability

Notify NOC of maintenance window

Perform fusion splices (target IL ≤ 0.10 dB each)

Install new enclosure and reseal per manufacturer spec

Run end-to-end OTDR trace on all 24 fibers

Confirm signal restore for all 14 affected tenant circuits

## Sign-Off & Completion

| Role | Signature | Print Name | Date |
| --- | --- | --- | --- |
| Technician Signature |  |  |  |

Fibey Field Ops  ·  Fiber Splice Restoration  ·  CONFIDENTIAL